# FAA Test Hazard Analysis

| FAR No.: 25.203 | FAR Title: Stall Characteristics | Risk Level: High |
| --- | --- | --- |
| Maneuver Title: Recovery Chute Functional Test |  |  |
| Maneuver Description: Ground test of stall/spin recovery chute. 1. Accelerate down the runway centerline. 2. At 60-80 kts, select idle power. 3. Deploy chute. 4. After inflation, jettison chute. |  |  |
| Hazard: Departing runway surface |  |  |
| Cause(s): 1. 1. Stall/spin chute may provide significant pitch or yaw moment 2. Chute may drag on runway when deployed |  |  |
| Mitigation(s): 1.1 1. Deployment test will be performed at heavy weight and forward center of gravity 2. Deployment test will be performed with crosswind less than 5 knots and on runway centerline 3. Chute deployment will be performed with engines in IDLE thrust to preclude plume impingement 4. Runway to be used will be surveyed 5. Runway will be swept of foreign objects after test. 6. Proper installation of chute canister and tail cone will be verified 7. Runway observers required |  |  |

Warning: It is imperative that this information be applied by a knowledgeable flight test professional given the inherently hazardous nature of these activities. This data is general and all items may not apply. Applicationmay require additional research to understanding the safe approach and mitigation sequence for a specific flight test situation.

FAA Note: The maneuver information described in this database is for reference only. Consult Title 14 CFR for actual FAR requirements.