# FAA Test Hazard Analysis

| FAR No.: 25.111 | FAR Title: Takeoff Path | Risk Level: High |
| --- | --- | --- |
| Maneuver Title: OEI Climb Gradient |  |  |
| Maneuver Description: See AC 25-7A Section 25.111 b High Altitude Installed Thrust Characteristics/Lapse Rate Take-offs & Thrust After 8 Seconds |  |  |
| Hazard: 1. Departure from runway |  |  |
| Cause(s): 1. 1. Loss of directional control 2. Inadequate runway length |  |  |
| Mitigation(s): 1.1 1. Engine cuts to idle 2.Minimum control speed evaluation (Vmca/Vmcg) completed prior to start of testing 3. Brief obstruction and escape route for airfield 4. Brief engine out safe speeds and techniques 5. Performance information to be available for all configurations 6. Verify adequate runway length for each take-off, particularly in the event of an actual engine failure 7. Power reduction speeds to be always greater than Vmcg 8. Perform tests in order of decreasing performance capability 9. Carry out test configuration build-up test point prior to performing any other test points in this procedure 10. Crew to wear protective gear and helmets. 11. All vehicles to be positioned at least 400 feet from runway centerline 12. Identify a suitable takeoff alternate airfield 13. Ensure adequate fuel to divert to alternate airfield prior to beginning tests. |  |  |

Warning: It is imperative that this information be applied by a knowledgeable flight test professional given the inherently hazardous nature of these activities. This data is general and all items may not apply. Applicationmay require additional research to understanding the safe approach and mitigation sequence for a specific flight test situation.

FAA Note: The maneuver information described in this database is for reference only. Consult Title 14 CFR for actual FAR requirements.